🚗  AutoShield-CAN IDS

Threat Analysis and Risk Assessment (TARA)

ISO/SAE 21434 Compliant

| Document Title | TARA — CAN Bus Intrusion Detection System |
| --- | --- |
| Project Name | AutoShield-CAN IDS |
| Author | Janani |
| Version | 1.0 |
| Date | 07 June 2026 |
| Classification | Confidential — Placement Portfolio |
| Standard | ISO/SAE 21434:2021 Road Vehicles Cybersecurity |

— Page Break —

# 1. Introduction

## 1.1 Purpose

This Threat Analysis and Risk Assessment (TARA) document identifies, analyses, and evaluates cybersecurity threats targeting the CAN bus network in a modern vehicle. It is produced in accordance with ISO/SAE 21434:2021 — the international standard for automotive cybersecurity engineering.

This TARA supports the AutoShield-CAN IDS project, a Python-based automotive Intrusion Detection and Prevention System (IDPS) prototype that simulates attack scenarios and detects them using a two-layer detection engine.

## 1.2 Scope

The scope of this TARA is limited to the 500 kbps powertrain CAN bus segment containing five ECU nodes:

| CAN ID | ECU Name | Function | Safety Level | Cycle Time |
| --- | --- | --- | --- | --- |
| 0x0C0 | Engine ECU | RPM, throttle, temp control | ASIL-D | 10 ms |
| 0x1A0 | Brake ECU | Brake pressure, ABS control | ASIL-D | 10 ms |
| 0x2B0 | Steering ECU | EPS angle and torque | ASIL-C | 10 ms |
| 0x1F0 | Transmission ECU | Gear, torque converter | ASIL-B | 20 ms |
| 0x4B0 | Body Control Module | Doors, locks, windows | QM | 100 ms |

## 1.3 Key Limitation Addressed

CAN 2.0 (ISO 11898) was designed in 1986 for closed, trusted networks. It has NO sender authentication, NO payload encryption, NO sequence numbers, and NO rate limiting. These four design gaps are the root cause of every threat identified in this document.

# 2. Asset Identification

## 2.1 Cybersecurity-Relevant Assets

An asset is any component, data, or function whose compromise could lead to safety harm, financial damage, or operational disruption. The following assets are identified on the CAN bus:

| # | Asset | Description | Type | Impact if Compromised |
| --- | --- | --- | --- | --- |
| A1 | Engine Control Data | RPM, throttle position, ignition state broadcast on 0x0C0 | CAN message | Vehicle speed manipulation, stalling |
| A2 | Brake Control Data | Brake pressure, ABS status on 0x1A0 | CAN message | Loss of braking, collision risk |
| A3 | Steering Control Data | EPS angle on 0x2B0 | CAN message | Loss of steering control |
| A4 | Door/Lock State | Lock/unlock commands on 0x4B0 | CAN message | Unauthorised vehicle access |
| A5 | CAN Bus Bandwidth | Shared 500 kbps medium | Communication resource | DoS — safety messages silenced |
| A6 | ECU Availability | All five ECU nodes operational | System resource | Safety system failure |

# 3. Threat Scenarios

Four threat scenarios are identified, each exploiting a specific design weakness in CAN 2.0. Each scenario is simulated by the AutoShield-CAN IDS attack engine and detected by the IDS detector module.

## 3.1  T1 — Replay Attack

| Threat ID | T1 |
| --- | --- |
| Threat Name | Replay Attack |
| Asset Targeted | A4 (Door Lock Data) |
| Attack Vector | Physical (OBD-II port) or Remote (compromised TCU) |
| Description | An attacker captures a legitimate door-unlock CAN frame (ID=0x4B0) and retransmits it at a later time. Since CAN has no sequence numbers or timestamps, the Body Control Module cannot distinguish a replayed frame from a genuine one. The vehicle unlocks without a valid key. |
| Safety Impact | Unauthorised physical access to vehicle. Theft of vehicle or contents. |
| Likelihood | High — relay attack kits are commercially available for under ₹10,000. |
| Severity | High |
| CVSS Score | 7.1 (High) |
| IDS Detection | Rule R2: IDS detects identical frame (same ID + same payload) appearing more than 3 times within a sliding window. |
| Mitigation | Implement message authentication codes (CMAC/HMAC) per ISO 21434 §15. Add monotonic counter to CAN payload. Use SecOC (Secure Onboard Communication) in AUTOSAR. |
| Residual Risk | High |

## 3.2  T2 — Fuzzing Attack

| Threat ID | T2 |
| --- | --- |
| Threat Name | Fuzzing Attack |
| Asset Targeted | A1, A2, A3 (Safety-Critical ECUs) |
| Attack Vector | Physical (OBD-II), Remote (compromised gateway) |
| Description | An attacker injects CAN frames with the IDs of safety-critical ECUs (Engine 0x0C0, Brake 0x1A0, Steering 0x2B0) but with random or boundary-value payloads. ECU firmware that lacks input validation may act on these values — e.g., interpreting a fuzzed brake pressure byte of 0xFF as 1020 kPa and applying full emergency braking at highway speed. |
| Safety Impact | Unintended braking, acceleration, or steering. Potential loss of vehicle control. Risk to occupant life. |
| Likelihood | Medium — requires bus access and ECU-specific knowledge. |
| Severity | Critical |
| CVSS Score | 9.1 (Critical) |
| IDS Detection | Rule R3: Payload range validation (e.g., RPM_raw > 28000 is physically impossible). Statistical detector: Z-score > 3.5 on payload bytes vs. trained baseline. |
| Mitigation | ECU firmware input validation. Payload range checks in IDS. Secure gateway filtering. AUTOSAR E2E Protection Library. |
| Residual Risk | Critical |

## 3.3  T3 — ID Spoofing Attack

| Threat ID | T3 |
| --- | --- |
| Threat Name | ID Spoofing Attack |
| Asset Targeted | A1, A2, A3 (ECU Identity) |
| Attack Vector | Remote (compromised ECU or TCU) |
| Description | An attacker sends CAN frames using the arbitration ID of a legitimate ECU — for example, impersonating the Engine ECU (0x0C0) with a payload reporting RPM=0 (engine stalled) while the vehicle is moving at speed. Dependent systems (dashboard, transmission, cruise control) act on false data. In the Jeep Cherokee 2015 attack, this technique was used to control brakes and steering remotely. |
| Safety Impact | False sensor readings. Incorrect ECU behaviour based on spoofed data. Safety system failure. |
| Likelihood | High — CAN has no sender authentication by design. |
| Severity | Critical |
| CVSS Score | 9.3 (Critical) |
| IDS Detection | Rule R1: Message rate doubles for a specific ID when the real ECU and attacker both transmit. Statistical detector: timing interval anomaly. Physical inconsistency detection (RPM=0 while speed>0). |
| Mitigation | SecOC with CMAC authentication per AUTOSAR. Hardware Security Module (HSM) in each ECU for key storage. Network segmentation with authenticated gateways. |
| Residual Risk | Critical |

## 3.4  T4 — Bus Flooding / DoS Attack

| Threat ID | T4 |
| --- | --- |
| Threat Name | Bus Flooding / DoS Attack |
| Asset Targeted | A5 (CAN Bus Bandwidth), A6 (ECU Availability) |
| Attack Vector | Physical (OBD-II) or compromised ECU |
| Description | An attacker floods the CAN bus with frames using ID 0x000 — the highest possible priority. CAN arbitration is deterministic: lower ID always wins. The flood frames win every arbitration contest, starving all legitimate ECUs. Safety-critical messages (ABS=0x1A0, airbag) cannot gain bus access. At 500 fps with ID 0x000, 10% of bandwidth is consumed by the attacker; at 5000 fps, the bus is completely saturated. |
| Safety Impact | Complete Denial of Service to all ECUs. ABS, airbag, and steering messages silenced during safety-critical events. |
| Likelihood | Medium — requires bus access but no protocol knowledge. |
| Severity | Critical |
| CVSS Score | 8.6 (High) |
| IDS Detection | Rule R5: Single ID accounts for >40% of bus traffic. Rule R1: Any ID exceeding 2.5× baseline rate. Statistical: timing interval collapses to near-zero. |
| Mitigation | Hardware CAN bus guardian (e.g., TJA1145 transceiver with bus-off protection). Rate limiting per ECU at gateway. Intrusion Detection trigger for bus-off recovery. |
| Residual Risk | High |

# 4. Risk Assessment Summary

Risk is evaluated using the ISO 21434 method combining Impact (severity of harm) and Feasibility (likelihood of attack success). Risk = Impact × Feasibility.

| ID | Threat | Impact | Feasibility | Risk Level | CVSS | Priority |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Replay Attack | High | High | High | 7.1 | P2 — Address |
| T2 | Fuzzing Attack | Critical | Medium | Critical | 9.1 | P1 — Immediate |
| T3 | ID Spoofing Attack | Critical | High | Critical | 9.3 | P1 — Immediate |
| T4 | Bus Flooding (DoS) | Critical | Medium | High | 8.6 | P1 — Immediate |

# 5. IDS Detection Architecture

## 5.1 Detection Layers

The AutoShield-CAN IDS implements two detection layers in accordance with defence-in-depth principles required by ISO 21434 §15:

| Layer | Method | Attacks Detected | Trade-off |
| --- | --- | --- | --- |
| Layer 1 | Rule-based (5 deterministic rules) | Replay, Flooding, Spoofing, Fuzzing (known patterns) | Zero latency. 100% recall on known attacks. Cannot detect novel attacks. |
| Layer 2 | Statistical anomaly (Z-score, training baseline) | Timing anomalies, payload drift, unknown attack variants | Requires training phase (100 frames/ID). Can detect zero-day variants. |

## 5.2 Detection Rules Implemented

| Rule | Name | Logic | Attack Detected |
| --- | --- | --- | --- |
| R1 | Frequency Rule | Message rate > 2.5× baseline for any CAN ID | Flooding, Spoofing |
| R2 | Duplicate Rule | Identical frame (ID+payload) seen ≥ 3× in window | Replay |
| R3 | Payload Range | Signal value outside physical engineering range | Fuzzing |
| R4 | Unknown ID Rule | CAN ID not in known ECU list appears on bus | Rogue node, Spoofing |
| R5 | Bus Domination | Single ID accounts for >40% of total bus traffic | Flooding |

## 5.3 Observed Detection Results

The following results were recorded during a single demo run of run_dashboard.py on the simulated 500 kbps CAN bus:

| Metric | Value | Metric | Value |
| --- | --- | --- | --- |
| Total frames analyzed | 6,350 | Trained ECU IDs | 6 |
| Total alerts generated | 3,800 | System uptime | 18.1s |
| REPLAY alerts | 221 | FLOODING alerts | 196 |
| FUZZING alerts | 3374 | SPOOFING alerts | 9 |

# 6. Mitigation Recommendations

The following mitigations are recommended per ISO 21434 §15 (Cybersecurity Controls) and UNECE WP.29 R155 requirements:

| ID | Threat | Mitigation | Standard Reference | Effort |
| --- | --- | --- | --- | --- |
| M1 | T1,T3 | SecOC (Secure Onboard Communication) — CMAC-AES128 authentication on safety-critical CAN IDs | AUTOSAR SecOC, ISO 21434 §15.4 | High |
| M2 | T1 | Monotonic counter in CAN payload — receiver rejects frames with stale counter values | ISO 21434 §15.4 | Medium |
| M3 | T2 | ECU firmware input validation — range-check all CAN signal values before processing | ISO 26262 ASIL software req. | Medium |
| M4 | T4 | Bus guardian at CAN transceiver (TJA1145) — hardware rate limiting per ECU node | ISO 11898-6 | High |
| M5 | All | Hardware Security Module (HSM) per ECU — key storage, crypto operations isolated from app code | ISO 21434 §15.6, SHE standard | High |
| M6 | All | Network segmentation — separate powertrain, chassis, and infotainment buses with authenticated gateway | UNECE WP.29 R155 | Medium |

# 7. Conclusion

This TARA has identified four high-to-critical cybersecurity threats targeting the CAN bus network of a modern vehicle. All four threats exploit fundamental design limitations of the CAN 2.0 protocol — the absence of sender authentication, payload encryption, sequence numbers, and rate limiting.

The AutoShield-CAN IDS prototype demonstrates that a two-layer Intrusion Detection System combining deterministic rule-based detection with statistical anomaly detection can effectively identify all four threat classes in real time. Detection results show alerts generated for all attack types within the training and detection phases.

Full mitigation requires hardware-level controls (SecOC, HSM, bus guardian) that are outside the scope of this software prototype but are referenced per ISO 21434 and AUTOSAR standards. This TARA serves as the threat modelling foundation for further automotive cybersecurity engineering work.

| Prepared by | Reviewed by |
| --- | --- |
| Janani Automotive Cybersecurity (Student Researcher) | — (Self-review, placement portfolio) |
| Date: 07 June 2026 | Version: 1.0 |